W.2	Inspect the property’s stormwater management system

A properly functioning stormwater management system enhances the property’s resilience, prevents pollution from runoff, and stops issues with flooding before they happen.

To be eligible for this point

- Use the checklist that follows to inspect the property’s stormwater management system.

- Take corrective action as necessary.

- Repeat inspections on a regular basis, especially as seasons change and after severe weather.

To earn this point

- Submit the completed checklist with your application.

| Stormwater management inspection checklist |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Vegetated areas |  | Ok |  | Not Ok |  | Action required |
| Slopes, embankments, and replant areas have sufficient growth |  |  |  |  |  |  |
| Riprap and other erosion mitigation measures present where necessary to prevent erosion |  |  |  |  |  |  |
| Downslopes of level spreaders and turnouts show no signs of erosion |  |  |  |  |  |  |
| Mowing performed no more than twice per year and at least six inches of vegetation remain |  |  |  |  |  |  |
| Parking lots and roads |  | Ok |  | Not Ok |  | Action required |
| Pavement clear of excessive sand and sediment |  |  |  |  |  |  |
| Pavement and gravel areas intact, not in disrepair |  |  |  |  |  |  |
| Gravel roads and shoulders graded; no accumulated sand or gravel creating false ditches |  |  |  |  |  |  |
| Ditches, swales, channels, and buffers |  | Ok |  | Not Ok |  | Action required |
| No obstructions, sediment, or debris |  |  |  |  |  |  |
| Ditch linings intact |  |  |  |  |  |  |
| Side slopes intact and not slumping |  |  |  |  |  |  |
| Riprap free of woody overgrowth; no underlying filter fabric showing |  |  |  |  |  |  |
| Culverts |  | Ok |  | Not ok |  | Action required |
| Inlets, outlets, and conduits free of debris; no obstructions to flow |  |  |  |  |  |  |
| No erosion damage at inlets and outlets |  |  |  |  |  |  |
| Catch basins |  | Ok |  | Not Ok |  | Action required |
| Grates free of debris and obstruction |  |  |  |  |  |  |
| Edges free of erosion damage |  |  |  |  |  |  |
| Retention and detention basins | Ok | Not Ok |  | Action required |  |
| Embankments free of settlement, slope erosion, slumping, and overgrowth |  |  |  |  |  |
| Seals, orifices, trash racks, and debris guards undamaged and free of obstruction |  |  |  |  |  |
| Control structure, impoundment, and forebay clear of accumulated sediment |  |  |  |  |  |
| Riprap spillways intact; no stones dislodged |  |  |  |  |  |
| Structure free of signs of hydrocarbons (e.g., motor oil) and other pollutants |  |  |  |  |  |
| Other stormwater management | Ok |  | Not Ok | Action required |  |
| Performance of system after heavy rain |  |  |  |  |  |
| Signs of runoff, pooling water, pollutants, erosion, etc. |  |  |  |  |  |
| Assess site for opportunities to install additional stormwater management features, such as permeable pavement and rain gardens |  |  |  |  |  |
| Third-party inspection and maintenance regularly performed as needed |  |  |  |  |  |

Sources: Maine Bureau of Land Resources, Minnesota Pollution Control Agency, City of Surprise, AZ, City of Stamford, CT

Alternative documentation
Instead of this form, you may submit the following to IREM®:

- Other stormwater management system inspection checklist

- Memo or invoice from third-party service provider